# PCTALKER Printer version - Hungarian - DEMO

Technical README: emulation of the original DOS printer-port D/A version

J.Kiraly

NVDA add-on version 0.1.5

# 1. Purpose of this add-on

This add-on is an NVDA reconstruction of the Hungarian PCTALKER Printer version. It demonstrates the audio quality of the version developed by J. Kiraly between 1989 and 1991.

The historical Printer version used an external D/A converter connected to the PC printer/parallel port. This reconstruction preserves the historical speech material and text-to-speech logic while replacing the obsolete hardware output stage with NVDA and the Windows audio system.

# 2. How the original DOS Printer version generated speech

The original program transformed Hungarian text through the PCTALKER dictionary and conversion logic into speech-element codes. OLVAS_P.ASM contains conversion and speech-element tables, RAWHUSR contains the historical speech sample data, and SZOTAR.TBL contains dictionary and exception substitutions.

After selecting the required speech elements, the DOS program sent successive multi-level sample values to an external digital-to-analog converter attached to the printer port. The D/A converter produced the analog voltage that was amplified and reproduced by a loudspeaker.

Unlike the PCTALKER PC Speaker version, the Printer version did not need one-bit PWM to simulate intermediate amplitudes. It therefore had no PC-speaker PWM carrier or PIT-based acoustic averaging stage.

# 3. CWI-2 / CP-HU character encoding

Version 0.1.5 corrects an important reconstruction detail: the text conversion now uses CWI-2 / CP-HU rather than CP852.

Unicode Hungarian text is explicitly converted to the CWI-2 byte positions expected by the historical PCTALKER tables before the original byte-oriented conversion logic runs.

For compatibility with the historical Printer-version tables, uppercase Ó, Ú, Ő and Í are mapped to their lowercase equivalents ó, ú, ő and í before encoding. Lowercase ő and ű now use their proper CWI-2 positions. The earlier ű -> ü workaround has been removed.

# 4. How the NVDA emulation works

NVDA supplies Unicode text. The Python engine expands numbers, applies SZOTAR.TBL substitutions, performs the reconstructed OLVAS_P.ASM conversion, retrieves the appropriate segments from RAWHUSR, and produces unsigned 8-bit mono PCM at a nominal 8,500 Hz.

NVDA's native WavePlayer sends that PCM to the selected Windows audio device. The modern sound system therefore replaces the physical printer-port D/A converter, while the historical PCTALKER speech material and conversion logic remain the basis of the sound.

Speech synthesis runs through a persistent worker with cancellation support, allowing new NVDA navigation speech to replace obsolete pending speech.

# 5. Speech rate and intonation

NVDA Rate 50 corresponds to the nominal 1.0x speed. Faster or slower settings resample the synthesized speech, changing duration and pitch in the same general manner as changing historical playback timing.

The reconstruction also retains the intonation behavior derived from the historical speech logic. Punctuation can affect local speech-element timing independently of the overall Rate setting.

# 6. Printer version versus PC Speaker version

The two PCTALKER branches share the general text-to-speech concept and historical Hungarian speech resources, but their output hardware was different.

The Printer version represented amplitude directly as multi-level sample values sent to an external D/A converter. The PC Speaker version represented amplitude indirectly through one-bit PWM and PIT timing, relying strongly on the internal speaker's physical response.

Consequently, Fast envelope, True PWM, Real PWM, PWM carrier noise and PC-speaker filter emulation belong to the separate PC Speaker demonstration. They are not part of this Printer-version add-on.

# 7. Simplified signal paths

Original DOS Printer version: Hungarian text -> SZOTAR/OLVAS conversion -> speech-element codes -> RAWHUSR samples -> timed printer-port output -> external D/A converter -> amplifier/speaker.

Current NVDA emulation: Unicode text -> CWI-2/PCTALKER conversion -> speech-element codes -> RAWHUSR samples -> unsigned 8-bit mono PCM at 8,500 Hz -> NVDA WavePlayer -> Windows sound system -> modern D/A converter -> speaker/headphones.

# 8. Version 0.1.5 changes

CP852 was replaced by explicit CWI-2 / CP-HU conversion.

The old lowercase ű -> ü substitution was removed.

Uppercase Ó, Ú, Ő and Í are mapped to ó, ú, ő and í for PCTALKER table lookup.

The synthesizer and add-on display name were changed to PCTALKER Printer version - Hungarian - DEMO.

The metadata now states: This addon demonstrates the audio quality of the Hungarian PCTALKER Printer version developed by J. Kiraly between 1989 and 1991.

# 9. Demonstration status

This add-on is a historical reconstruction and demonstration build. Its purpose is preservation and demonstration of the characteristic sound and operating principles of the Hungarian PCTALKER Printer version on modern Windows/NVDA systems.

# 10. Technical summary

| Item | PCTALKER Printer version - Hungarian - DEMO 0.1.5 |
| --- | --- |
| Historical hardware model | External D/A converter connected to printer/parallel port |
| Historical speech data | RAWHUSR |
| Conversion/table source | OLVAS_P.ASM |
| Dictionary | SZOTAR.TBL |
| Historical text encoding | CWI-2 / CP-HU |
| Compatibility mappings | Ó->ó, Ú->ú, Ő->ő, Í->í |
| Synthesized audio | Unsigned 8-bit mono PCM |
| Nominal sample rate | 8,500 Hz |
| NVDA playback | Native NVDA WavePlayer |
| PWM carrier | None |
| Language | Hungarian (hu_HU) |